Project 1: Op Amp Design

Process:	MOSIS ON 0.5 um

Supply:	0V and 5V

Architecture:	fully differential two stage, with folded cascade first stage and a single CMFB

1st stage:	PMOS input for small noise, gm control by source degeneration

2nd stage:	CS amplifier with NMOS input for large gm

CMFB:	passive CM detector, diff-pair with NMOS input and PMOS diode load

Bias:	PMOS Widlar Iref generator with a single resistor, current mirror bias

Comp:	lead compensation

Power budget:	<= 2mA * Vdd

Load cap:	2 pF on each output

Specifications:

Large signal:	OSR (output swing range): 10% to 90% Vdd for both Vo+ and Vo-

OSR: -85% to +85% for Vod

ICMR (input common mode range): 0% to 70% Vdd

Slew Rate: SR+, SR- >=100 V/uV, at both Vo+ and Vo-

SR+, SR- >=200 V/uV, at both Vod

Small signal:	Avd(0) > 90 dB

GBW >= 30MHz

Small step response percentage overshot: <= 15% for gain=-1

(Phase Margin at unity gain frequency of A: >=55o)

0.1% settling time < 200 ns for small step response

Circuit schematics: see ppt. Modify as you wish.

Tasks: 		Size all transistors, Rss, Rc, Cc, R and C.

Simulate to validate your sizing