Project 2: Flash ADC Design

Process:	MOSIS ON 0.5 um

Supply:	0V and 5V

Ain range:	>= 2.5V, with Vreftop and Vrefbot to be defined by designer

Resolution:	6 bits

INL:		±0.75 LSB

DNL:		±0.5 LSB

Speed:		Not specified, but should be > 4 MSPS, target 20 MSPS

Power:		Not specified, but should be < 450 mW

Area:		Not specified, but should be < 1 mm2

Architecture:	R-string + Comparator array + encoder

Required pins:

AVDD, AGND

DVDD, DGND

VREFT, VREFB

Ain

D1-D6, D1=LSB, D6=MSB

CLK

OE (output enable)

Design and Performance to be verified:

Brief description of basic operation, key components, key constraints, and sizing decisions

Power, area, speed trade-off decisions

DC analysis of all key node voltages and branch currents

Transient analysis of key node voltages in key building blocks

Converter Static Performance: INL, DNL, gain error, offset

Converter Dynamic Performance: SFDR, SNR, THD, SINAD, ENOB at 3 frequencies: near Nyquist, near ¼ Nyquist, and near 5% Nyquist.