EE 475 Fall 2009 Quiz #3

Name:		Solution

- If h(t) is the impulse response of a system, the system is BIBO stable iff 						   is finite						.

- Assume no pole/zero cancelation. A system is A.S. if its poles	all have negative real parts	.

- a*s2 +b*s+c is A.S. if 	a, b, c all have the same sign	.

- a*s3 +b*s2+cs+d  is A.S. if 	a, b, c, d all have the same sign and b*c > a*d	.

- G1*G2

- G1+G2

- G1/(1+G1G2)

- G2, and 1/G2