EE 475 Fall 2009 Quiz #4

Name:	Solution

- Assume no pole/zero cancelation. A system is A.S. if its poles	all have Re<0	.

- a*s2 +b*s+c is A.S. if 	a, b, c have the same sign	.

- a*s3 +b*s2+cs+d  is A.S. if 	a, b, c have the same sign and b*c > a*d	.

- G1/(1+G1G2)

- n1d2/(n1n2+d1d2)

- n1/(n1+d1)

- For the block digram below

- List the forward path gains:	H1H2H3					.

- List the loop gains:	-H1H5, +H2H6, -H3H7				.

- List the non-touching loop pair’s gain product:	H1H3H5H7		.

- The determinant of the graph is =	1+H1H5+H3H7-H2H6+H1H3H5H7	.