EE 475 Fall 2009 Quiz #7

Name:		Solution

- For a unity feedback control system with open loop transfer function G(s), give the definition of:

System type:	#integrators in G(s) or # factors of s in denominator of G

Position error constant	G(0), or b0/a0, or lim of G(s) as s  0

Velocity error constant	sG(s) at s=0, or b0/a1 when a0=0, or lim of sG(s) as s  0

Acceleration error constant	s^2G(s) at s=0, or b0/a2 when both a0 and a1=0, or lim of s^2G(s) as s  0

- Given the system below:

What is the system type with respect to reference input r(s)?	2

What is the system type with respect to disturbance input d(s)?	1

What is the steady state error due to a step r(s)?	0

What is the steady state error due to a ramp r(s)?	0

What is the steady state error due to a step d(s)?	0