EE 475 Fall 2006 Final Exam

Name

Instructions:

- This is a closed-book and closed-notes exam for individual work. You may have two sheets of formulae of size no larger than US letter. Calculator is allowed.

- Time for the exam is 120 minutes.

- Do all problems. The points for each problem are marked in the parenthesis after the problem. Total points are 100.

Problems:

- Consider a linear control system given by the following state space model.

x=−1000−2000−3x+210uy= 1 −1 1 x

  - Determine the characteristic values (eigenvalues) of the system. (3 pts)

  - Determine the state transition matrix eAt. (5 pts)

- Consider the same system given in problem 1.

  - Determine the transfer function from u to y and its poles and zeros. (3 points)

  - Determine the internal stability and I/O stability of the system. (3 points)

- Determine the controllability and observability of the system given in problem 1. (6 points)

- For the system given in problem 1, give a new 3rd order state space representation in controllability canonical form and one in observability canonical form. (6 points)

Find the transfer function from r to y and the transfer function from e to y. (10 pts)

G3

H1h11

G1

G2

G4

H2

y

r

e

+

+

+

+

+







- Consider the system below and let p=0.

  - Determine conditions on K and z so that the system is stable. (6 points)

  - Determine all possible conditions on K and z so that the system will have sustained oscillation. Find the oscillation frequency. (8 points)

- Hand sketch the root locus of   1 + KG(s) = 0 as K varies from 0 to +∞, where

G(s)=s+2s(s+1)(s+3)2

Make sure you provide verbal description on the following: open-loop pole/zero map; real axis decision; asymptotes; estimation of breakaway points; and jw-axis crossing. (12 points)

- For the system given in problem 6, is it possible to select an appropriate K value to achieve the following step response specification? ts <= 8 sec for 2% tolerance; Mp <= 10%; and tp <= 3 sec. If possible, determine such a K. If not possible, suggest an appropriate controller type and explain why it can work. You don’t need to numerically design the controller. (10 points)

- The open-loop transfer function of unity feedback control system is tested experimentally to obtain its frequency response. The data is presented in the following Bode plot.

- Find the gain crossover frequency, phase crossover frequency, gain margin, and phase margin, and mark them on the graph. (3 pts)

- Determine the closed loop stability. (1 pts)

- Estimate the percentage overshoot, settling time, and rise time in the closed-loop unit step response. (5 pts)

- Determine the system type, estimate the velocity error constant and the steady state tracking error in the closed loop unit ramp response. (3 points)

- Estimate the bandwidth, resonance frequency, and resonance peak of the closed-loop frequency response. Hand sketch the closed-loop magnitude response and label BW, r and Mr on the graph. (6 points)

- In a unity feedback set-up, the open-loop transfer function G(s) is stable and its Nyquist plot for >0 is given below.

- Draw the complete Nyquist plot, label w=0+, +∞, –∞, 0–, and arrows for increasing frequency. (3 pts)

- Determine the number of encirclement of the “-1” point, the number of unstable closed-loop poles, and the closed-loop stability. (3 pts)

- Make appropriate additions to the Nyquist plot and use these to estimate the gain margin and phase margin. (4 pts)

- By what ratio can the system’s overall gain be increased or reduced before losing closed-loop stability? (2 pts)