EE 475 Fall 2007 Midterm Exam #2

Name

Instructions:

- This is a closed-book and closed-notes exam for individual work.

- You may have two sheets of formulae of size no larger than US letter.

- Calculator is discouraged.

- Time for the exam is 50 minutes.

- Do all problems, but easier ones first, since all are equally weighted.

Problems:

- For the system given below, estimate the values of K and Kt so that a maximum percentage overshoot of 9.6% and a settling time of 0.05 sec for a tolerance band of 1% are achieved.

- For the following closed loop transfer functions, determine whether the system is marginally stable, asymptotically stable, or unstable. (no pole/zero cancelation happened)

a)	M(s)=5s2+6s+3(s+2)(s−2)  :

b)	M(s)=(s−3)(s+4)(s+2)(s+5)  :

c)	M(s)=10s2+6s+1s(s+100) :

d)	M(s)=(s+1)s2(s+2)        :

- Using the Routh-Hurwitz criterion, determine the number of roots of each polynomial that are in the open right half plane.

a) d(s)=s4+s3+6s2+2s+1

b) d(s)=2s4+s3+4s2+5s+12

- Determine the step, ramp, and parabolic error constants of the following unity-feedback control systems.  The forward-path transfer functions are given.  Assume the closed loop systems are stable.

a) G(s)=50(1+s)(1+20s)

Kp=			, Kv=			, Ka

b) G(s)=2s(s+1)(s+2)

Kp=			, Kv=			, Ka

c) G(s)=5s+1s2(s2+5s+6)

Kp=			, Kv=			, Ka

- For the systems in the above Problem, find the steady state error values due to a unit step input, a unit ramp input, and a unit acceleration input.

a) ess_step =			, ess_ramp=			, ess_acc=

b) ess_step =			, ess_ramp=			, ess_acc=

c) ess_step =			, ess_ramp=			, ess_acc=

- Given the following prototype 2nd order systems, answer the questions below.

a) 	400s2+20s+400			b)	1600s2+40s+1600

c)	100s2+10s+100			d)	900s2+20s+900

Which system’s transient decays the fastest? 	a	b	c	d

Which system has the highest percent overshoot? 	a	b	c	d

Which system has the longest rise time? 		a	b	c	d

Which system has the longest settling time? 		a	b	c	d

- Sketch the s-domain diagram that corresponds to the following specifications for a prototype 2nd order system.

a)	Percent overshoot less than or equal to 5%

b)	Rise time less than or equal to 1.8 second

- Sketch the s-domain diagram that corresponds to the following specifications for a prototype 2nd order system.

a)	Percent overshoot less than or equal to 5%

b)	Settling time less than or equal to 4 second (tolerance band is ±2%)

- Sketch the s-domain diagram that corresponds to the following specifications for a prototype 2nd order system.

a)	Settling time less than or equal to 10 second (tolerance band is ±2%)

b)	Peak time less than π seconds

- Given the forward-path transfer function of a unity-feedback control system, quickly sketch the root locus for K ≥ 0.  Do not calculate the jw-axis crossings or departure/arrival angles.  Approximately locate any breakaway points.

G(s)=K(s+3)(s+1)(s+2)(s+4)(s+5)

- Using the Routh-Hurwitz method find the values of K that make the system stable, the value of K that yields sustained oscillation, and the oscillation frequency.

- Sketch the root locus of the system in the previous problem for K>=0. Make sure your real axis and asymptotes are correct. Estimate breakaway points.

- When K = 10, the system below is stable and the steady state error due to ramp input is 0.5.  Find the range of K which results in the system being asymptotically stable.

- The step response of a second-order prototype system is plotted below.

3.14 sec

Time

Find yss, ess to step, ymax, Mp, and the transfer function of the system.

- The forward-path transfer function of a unity-feedback control system is given as:

The root-locus diagram has been constructed for this system and is shown below.

A

A*

Find the value of K at A/A*, and the value of A.