EE 475 Fall 2006 Quiz #6

Name:

- Given the transfer function of a system, we know that the system is BIBO stable if 		all poles have strictly negative real parts

- Given the A matrix of a system, we know that the system is asymptotically stable if A has eigenvalues that 	have strictly negative real parts

- If a system is asymptotically stable, what can we say about its BIBO stability? 		It is BIBO stable

- If a system is completely controllable, completely observable, and not asymptotically stable, then the system is BIBO 	unstable

- A system has no pole/zero cancellation and it is BIBO stable. Based on this we can determine the system’s internal stability: it is 	asymptotically stable

- The regular Routh criteria have two parts: a) if 	1st col have same sign 	, then all roots have negative real parts; b) the number of roots in the right half plane is equal to	the number of sign changes in 1st col

- Based on the Routh criteria, a second order polynomial is Hurwitz (ie stable) if and only if 	all coeff have same sign

- Based on the Routh criteria, a third order polynomial is Hurwitz (ie stable) if and only if two conditions are met: a) 	all coeff have same sign				; and b) 	b*c > a*d

- In special case 1, we have to use a so-called auxiliary polynomial. This case happens when 	a whole row = 0	 in constructing the Routh table, and in this case the roots of the auxiliary polynomial are 	also roots of original polynomial

- In the other special case when the first element in a row is zero while the whole row is not zero, we replace the 	0	 by 		 and continue as usual.