EE 475 Fall 2006 Quiz #8

Name:

The closed-loop step response of a system is given below. Mark, using lines and arrows, the following specs on the graph and write the numerical value by the marks: yss, ess, ymax, Mp(%), td, tr, ts, and tp. Tolerance band = 2%.

Bonus: system type=		, Kp=		, Kv=		, Ka=

Consider the following system and assume that parameters are selected so that closed loop system is asymptotically stable.

The system type with respect to r(s) = 	, Kp=		,Kv=

If r(s) = 1/s, d1=d2=0, the steady state value of e(t) =

If r(s) =t*us(t), d1=d2=0, the steady state value of e(t) =

The system type with respect to d1(s) =

If r(s) =0, d1=1/s, d2=0, the steady state value of e(t) =