EE 475 Fall 2006 Quiz #9

Name:	Sol

- The regular Routh criteria have two parts: a) if 	all #’s in the 1st col have the same sign, then all roots have negative real parts; b) the number of roots in the right half plane is equal to	the # of sign changes in the 1st col.

- Based on the Routh criteria, a second order polynomial is Hurwitz (ie stable) if and only if 	all coeff > 0 or all coeff have the same sign

- Based on the Routh criteria, a third order polynomial is Hurwitz (ie stable) if and only if two conditions are met: a) 	all coeff have the same sign (or >0)	; and b) 	b*c>a*d, or prod of mid two coeff > prod of outer two coeff

- We know that a system is BIBO stable but don’t know anything else, is this system asymptotically stable?	Can’t tell

- We know that a system is NOT BIBO stable but don’t know anything else, is this system asymptotically stable?	No

- We know that a system is asymptotically stable but don’t know anything else, is this system BIBO stable?	Yes

- A system’s unit step response has steady state value yss=0.95, what is ess?	0.05

- What is the system type of the system in 7? 	0

- That system has Kp=	19	, Kv=	0	, Ka=	0

- Its ess to ramp = 	infinity		, ess to acc = 	infinity