EE 475 Fall 2006 Quiz #10

Name:

- Which of the following most directly affects the settling time?

  - Real part of the dominant pole

  - Imaginary part of the dominant pole

  - Magnitude of the dominant pole

  - Angle of the dominant pole made with the negative real axis

- Which of the above 4 choices most directly affects the percentage overshoot?

- Which of the above 4 choices most directly affects the oscillation frequency?

- Which of the above 4 choices most directly affects the rising time and delay time?

- Mark true or false for the following:

Decreasing while keeping nconstant will increase the overshoot.

Increasing n while keeping constant will decrease the rise time.

Decreasing  while keeping nconstant will increase the settling time.

+

2s+80

s2+10s+20

- The obtain the step response for the above system, I use matlab command step(num, den), where num = [	,  	] and den = [	  ,  	,  	].

- The system type of the system is 		.

- The error constants are Kp = 	, Kv = 		, Ka = 		.