EE 475 Fall 2006 Quiz #12

Name:

- Match the following controller transfer functions with the correct names.

- 10				      e	1.   Lead

- 10s+20			      d	2.   Lag

- 100(10s+1)/s		      g	3.   lead-lag

- 2(s+5)/(s+1)		      a	4.   P (proportional)

- 10 (s+1)/(s+10)		      b	5.   PD (proportional plus derivative)

- (2s+1)(s+10)/s		      c	6.   PI (proportional plus integral)

- (s+1)(s+100)/(s+10)^2	      f	7.   PID (prop. plus int. plus derivative)

- 10/(s+10)			      h	8.   None of the above

- When the root locus does not pass through the desired region for the closed loop poles, what type of lowest order controllers should be used? 	Lead or PD

- When the steady state tracking error needs to be reduced by a factor of 5 while other specifications are all OK, what type of controller should be used? 	lag

- Out of P, PD, PI, lead, and lag, which can be used to move the closed-loop poles into a desired region: 	P, PD, lead	, which can be used to reduce steady state tracking error: 	P, lag, PI	, which can be used to increase the system type so as to eliminate a nonzero steady state error: 	PI	, which may cause sluggish settling behavior:	PI, lag	, which is sensitive to high frequency noise: 	PD	.