EE 475 Fall 2006 Quiz #13

Name:		solution

- A system’s open-loop transfer function Bode plot is given below. Fill in the blanks:

  - The gain cross-over frequency is 		~8-9	 rad/sec.

  - The phase cross-over frequency is 	none	 rad/sec.

  - The gain margin is 	infinity		 dB.

  - The phase margin is 	~40-45		 degrees.

  - The system’s closed-loop stability is 	stable		.

  - System type = 	1	.

  - Position error constant =		infinity		.

  - Velocity error constant = 		~40dB or 100		.

  - Acceleration error constant =	0	.

  - Steady state error to a unit step input = 		0	.

  - Steady state error to a unit ramp input = 		~0.01	.

  - Steady state error to a unit acceleration input = 	infinity		.

- A system’s open-loop transfer function Bode plot is given below. Fill in the blanks:

  - The gain cross-over frequency is 		~2	 rad/sec.

  - The phase cross-over frequency is 	~0.9	 rad/sec.

  - The gain margin is 	~negative 20 to 25 	 dB.

  - The phase margin is 	~negative 20	 degrees.

  - The system’s closed-loop stability is 	unstable		.

  - System type = 	2	.

  - Position error constant =		infinity		.

  - Velocity error constant = 		infinity		.

  - Acceleration error constant =	~1	.

  - Steady state error to a unit step input = 		infinity			.

  - Steady state error to a unit ramp input = 		infinity			.

  - Steady state error to a unit acceleration input = 		infinity			.