EE 475 Fall 2010 Final

Part 1: Controller Design in Matlab

A motion control system has plant transfer function

Gp(s) = 156250/{s(s2+625s+156250)}.

The design specifications are:

- Ess to step input is 0

- Ess to ramp input is <= 0.1%

- Step response rise time tr <= 7 ms

- Settling time for 3% tolerance ts <= 15 ms

- Maximum overshoot Mp between 3% and 10%

- Closed-loop resonance peak Mr <= 0.05 dB

- Closed-loop bandwidth BW >= 50 Hz (=100 pi rad/sec)

Documents to be submitted (all electronic):

- An m-file that I can just run without any user input

- A pdf or word file containing evidence that all specifications are met

- A word file with verbal description of your thinking process and design decision making

Due time: email must be time stamped before 11:59pm on Friday, 12/10.