EE 475 Fall 2011 HW #3

Due: 9/14/2011

- In Lecture 7 on slides 3 and 4, We assumed that Kt*R2/R1=1, R4=R2, and R3=R1. With these assumptions, verify that the op amp circuit and the v1 to v2 block on slide 3 reduce to the summing junction and the gain block on slide 4. Then find the input output transfer function for the system on slide 4.

- Modify the op amp circuit on slide 3, which is a proportional controller, by inserting a capacitor in series with R2. Show that the resulting op amp circuit is a PI (proportional plus integral) controller. Show that there is an appropriate capacitor voltage to provide any desired biasing voltage v10. Find the right capacitor voltage corresponding to v10=1.5V.

- With the modified op amp circuit as in the previous problem and the biasing voltage for v1 as a symbol v10, replace the v1 to v2 block by a gain. (The derivative evaluated at v1 = v10 symbolically.) Find the transfer function from vin to .

- Now introduce another change. Add an integrator (1/s) block after so that the final output is . The feedback block is now connected from instead of  Replace Kt by Kp. Use the PI op am circuit from 3, find the transfer function from vin to .

- Slides 5 to 8 provided the detailed derivation of a state space model of an armature controlled DC motor. Now suppose the motor is a field controlled motor. Re-do the derivation to get a state space model for the field controlled DC motor.

- On page 72, take M=10, m=1, l=1, g=10 in eq(3-22) and (3-23). Enter the state space model into Matlab. Obtain the transfer function from u to y1 and y2. Find the poles of the system.

- Consider the transfer function given in eq(3-7). Expand the denominator into a polynomial of s in descending order. Give a state space model for this transfer function. Use the simplest form as given on slide 20.

- Re-do prob 1 part b and part c from HW1.

- Do B-2-12

- Do B-2-13. Linearize at the center: x0=3, y0=11. What is z0?